TRY OUT MODUL LINUX ENVIRONMENT

IT NETWORK SYSTEMS ADMINISTRATION

Login

Username: root/user

Password: P@ssw0rd123

System Configuration

Timezone: Europe/Paris

General Configuration

| Fully Qualified Domain Name | Ipv4 | Services |
| --- | --- | --- |
| fw.smkn22.id | internet: 100.100.100.100/24 int-zone: 192.168.10.10/24 dmz-zone: 172.16.10.10/24 | Firewall(nftables), VPN Server(WireGuard) |
| int-srv.smkn22.id | int-zone: 192.168.10.254/24 | DNS Server(Bind9), LDAP Directory(slapd), Certificate Authority(OpenSSL) |
| mail-srv.smkn22.id | dmz-zone: 172.16.10.251/24 | Mail Server (Postfix(SMTP), Dovecot(IMAP)), Webmail(Roundcube), Mailclient (Thunderbird) |
| web-01.smkn22.id | dmz-zone: 172.16.10.252/24 | Virtual IP(keepalived), HAProxy, Web Server |
| web-02.smkn22.id | dmz-zone: 172.16.10.253/24 | Virtual IP(Keepalived), Load Balancer(Haproxy), Web Server(Nginx) |
| ani-clt.smkn22.id | internet: 100.100.100.200/32 | Client |

| NAME | NETWORK |
| --- | --- |
| int-zone | 192.168.10.0/24 |
| dmz-zone | 172.16.10.0/24 |
| internet | 100.100.100.0/24 |
| vpn | 10.10.10.0/24 |

Topology

# Part 1 INT (Internal)

Int-srv.smkn22.id

# DNS

Set up a DNS server using BIND9 to manage domain names.

- Install & Configure BIND9

- Create DNS Zones

  - Create a forward zone for: smkn22.id

  - Create a reverse zone for the int-zone and dmz-zone

- Add DNS Records

  - Add forward and reverse records for all servers listed in the general configuration.

  - Add additional records on the table.

| Type | Name | Value/Target |
| --- | --- | --- |
| A | www | 100.100.100.100 |
| A | mail | 172.16.10.251 |
| A | vip | 172.16.10.200 |
| A | vpn | 100.100.100.100 |
| CNAME | www.int | vip.smkn22.id |
| MX | @ | 10 mail.smkn22.id |

## LDAP

Install and configure LDAP using slapd to manage user accounts. These users will be used to log in to the mail server.

- Install LDAP and use slapd

- Create LDAP users

● 	Create the following users in LDAP. These users will be used for email login (authentication)

| Full Name | Username (uid) | Password | Organizational Unit (OU) | Email Address |
| --- | --- | --- | --- | --- |
| Administrator | admin | P@ssw0rd | - | - |
| FuFuFaFa | fufufafa | P@ssw0rd | qwerty | fufufafa@smkn22.id |
| wowo1 wowo2 … wowo5 | wowo1 wowo2 … wowo5 | P@ssw0rd | qwerty | wowo1@smkn22.id wowo2@smkn22.id … wowo5@smkn22.id |

## CA

Use OpenSSL to create your own Certificate Authority (CA) and generate certificates for your server (web and mail).

1. Create Root CA

- Make a Root CA with the common name: “SMKN22-CA”

- For other fields like Country, State, etc., you can use any value

- Install SMKN22-CA on every server and client

2. Create Certificates for Servers

- Generate and sign the following certificates directly using the Root CA:

- A certificate for web server, valid for: www.smkn22.id, www.int.smkn22.id

- A certificate for mail server, valid for mail.smkn22.id

2. Save All Certificates

- Save the certificates in this folder: /opt/grading/ca

- ca.pem: Root CA certificate

- web.pem: Web server certificate

- mail.pem: Mail server certificate

## Ansible

We will set up Ansible on the server int-srv and make it control server web-02. All Ansible files (settings, playbooks, and modules) must be saved in the /etc/ansible directory.

- Configure the Ansible inventory to include web-02, so you can test the connection using the following command:

ansible web-02 -m ping

- Create playbook 01-web-int.yml

  - This playbook will install Nginx

  - It will also create a virtual host that listens on port 8000

  - When you open the website, it will show the message: “Awikwok bet!”

  - Make the playbook idempotent

  - To run the playbook, use this command:

ansible-playbook /etc/ansible/01-web-int.yml

# Part 2 DMZ (Demilitarized Zones)

mail-srv.smkn22.id

### Mail Server

Deploy a fully functional mail server using Postfix and Dovecot to allow users to send and receive emails under the domain smkn22.id.

- Software Installation

  - Postfix - to manage sending and receiving emails SMTP.

  - Dovecot - to enable users to access and read emails via IMAP.

- Mail Server Configuration

  - Configure the server to handle mail delivery for the domain smkn22.id

  - Users should be able to read their emails using IMAP

  - Enforce TLS encryption for all connections between email clients and the server

    - If the Certificate Authority (CA) task has been completed, use the certificate issued for the mail server.

    - If not, generate a self-signed certificate.

  - Ensure clients that trust the SMKN22-CA can validate the certificate.

- LDAP Integration

  - Integrate with LDAP to authenticate users via int-srv.smkn22.id.

  - Set up Maildir directories for the following users:

    - fufufafa

○ 	wowo1 through wowo5

  - Ensure that the LDAP user fufufafa can

    - Login in using ther UID (username).

  - Restrict login access:

    - Only users under ou=qwerty are permitted to authenticate and use the mail server.

- Webmail Access

  - Install Roundcube using Apache2 as the web server

  - Expose the Roundcube webmail interface on:

    - https://mail.smkn22.id

- Setup e-mail  fufufafa@smkn22.id in Thunderbird, and can send to other users/himself

web-01.smkn22.id & web-02.smkn22.id

### HA Web

Set up a high-availability reverse proxy system using HAProxy and Keepalived on two servers: web-01 and web-02.

- Keepalived Configuration (Virtual IP)

  - Create a virtual IP (172.16.10.200/24) that will automatically move to the backup server if the main server goes down.

  - web-01: acts as the MASTER with priority 120

  - web-02: acts as the BACKUP with priority 110

- HAProxy Configuration for Load Balancing

  - Install and configure HAProxy on both servers (web-01 and web-02) to distribute traffic to two Nginx web servers:

  - web-01: 172.16.10.252:8080

  - web-02: 172.16.10.253:8080

  - All HTTP requests must be redirected to HTTPS.

  - HAProxy should perform TLS termination (HAProxy handles HTTPS and forwards to Nginx using HTTP).

  - Add an HTTP header via-proxy: hostname to responses (replace “hostname” with the proxy’s hostname) to help with troubleshooting.

  - If you already have a certificate from your CA, use the web certificate (web.pem).

  - Make sure a client that only trusts your root CA can validate the entire certificate chain.

  - If not, create a self-signed certificate.

- Web Server nginx

  - On web-01 and web-02 install nginx and listen port 8080 with index.html “Hello from ${hostname}”

# Part 3 Firewall

fw.smkn22.id

### nftables

Create firewall and NAT rules using nftables to control which traffic is allowed or blocked in your network.

- Allow Internet Access

  - Devices in the INT (internal) and DMZ networks can access the internet.

- NAT Masquerade

  - When sending traffic from your network to the internet (through the WAN), the firewall must replace the source IP with its own WAN IP using masquerade NAT.

- Port Forwarding

  - Forward incoming traffic from the internet to servers in the DMZ:

| Port | Protocol | Forward To |
| --- | --- | --- |
| 80 | TCP | 172.16.10.200 |
| 443 | TCP | 172.16.10.200 |
| 53 | TCP/UDP | 192.168.10.254 |

- VPN Access

  - VPN clients should be able to reach both internal and DMZ networks.

- Mail Server to LDAP

  - The mail server must be allowed to query the LDAP service running on int-srv.

- Block Everything Else

  - Any other traffic that’s not mentioned above should be blocked (default deny).

### Wireguard VPN

Set up WireGuard as a VPN server so that an external workstation (client) can connect securely to the internal network.

- Secure VPN Connection

- Use WireGuard to create a secure tunnel from the client to the internal network.

- All internet traffic from the client should be routed through the VPN tunnel.

- Pre-shared Key (PSK)

- For extra security, add a pre-shared key to the tunnel (used along with public/private keys).

- DNS Configuration

- The client should use the internal DNS server (not public DNS) to resolve hostnames while connected.

- IP Addressing

- Assigns VPN to the 10.10.10.0/24 subnet

- Server wg0: 10.10.10.1/24

- Client wg0: 10.10.10.2/24

- Save & Enable Configuration

- Save the WireGuard config as /etc/wireguard/wg0.conf

- Start it using: sudo wg-quick up wg0

- Enable it to run at boot (system service): sudo systemctl enable wg-quick@wg0

# Part 4 Client

General Configuration

1.    Create a local user named fufufafa

- Set the password to P@ssw0rd

2.    Configure VPN wireguard to fw.smkn22.id

- Save the WireGuard config as:

/etc/wireguard/wg0.conf

Start it using:

sudo wg-quick up wg0

- Enable it to run at boot (system service): sudo systemctl enable wg-quick@wg0

3.    Ensure access to internal web services via vpn

- Able to access https://www.smkn22.id and https://www.int.smkn22.id

4.    Verify webmail access through VPN

- Access Roundcube webmail at https://mail.smkn22.id after VPN connection

- Successfully log in using the account fufufafa@smkn22.id

- Able to send an email to the same account (self-mail test) without issue

5.   https://www.smkn22.id is defined as startpage, can be reached and doesn't show any certificate errors